Homework Project 2

Questions that must be answered in RED

For this homework assignment we will use the Simulink model and accompanying code of Bradley Sutton, PhD, University of Illinois, that was adapted from the MATLAB code presented in Hoppensteadt and Peskin , "Modeling and Simulation in Medicine and the Life Sciences" and subsequently modified in part by Claude Lieber, MD. Answer all questions and parts that are in red.

- Use the Simulink LEFT HEART AND AORTA model and associated Matlab files given on Canvas. The Simulink model (provided) must be run through the MATLAB .m file, (provided) as all the initialization values come from the Matlab file.

  - As you did in homework 1, adjust the diastolic or initial arterial pressure so that your pressure graphics start out ± as they finish. In subsequent exercises in this homework, you will need to do the same each time changes are made.

    - Run the program, study the plots, show the plots in your submission:

    - Flows through the aortic valve, mitral valve, and the venous flow

    - Ventricular compliance

    - Cardiac cycle

    - Left ventricular pressure and systemic artery pressure measurements

    - Relationship between systemic artery pressure and systemic artery volume

- In the emergency department and in the operating room, we initially treat a low blood pressure of any etiology with infusions of isotonic saline, blood products or both depending on the circumstances.

  - From the plots obtained so far explain the rationale for this treatment.

- Inspect the blood flows obtained.

  - Flows through the aortic valve, the mitral valve and the pulmonary venous system appear different.

    - Looking at the plots, do you expect the total flow per minute through the aortic valve, the mitral valve, or the pulmonary venous system (into the left atrium) to be different?

    - Describe and explain the differences, i.e.

      - differences in when the flow starts and finishes during each cardiac cycle,

      - in length of time of flow during each cardiac cycle, maximum flows of each ,

      - and the total flow of each per minute.

    - State the ONE aspect of these flows that must be the same for the pulmonary artery flow, the mitral flow and the aortic flow per minute. No exact calculations necessary for this question. Discuss qualitatively rather than quantitatively.

  - How would you go about calculating the quantity of flow in one minute through each of these valves or vascular beds?

  - How much is it. Show code (hint: it’s just several of lines of code).

  - What name do we assign to this amount of blood flow? Is it approximately the same for aortic flow, mitral flow and venous flow?

- The pressure profile of the left ventricle and the systemic arterial circulation differs.

  - How do these differ and explain why? In other words, explain why there exists a diastolic pressure in the arteries that is markedly different from that in the ventricles.

- Examine the cardiac cycle (one of the plots) and submit the relevant plot.

  - On your plot indicate (with specific indicators to be sure to pinpoint the areas)

    - where diastolic filling of the ventricle occurs,

    - where the mitral valve opens & where it closes,

    - The point that denotes the end diastolic volume and pressure.

    - The point that denotes the end systolic volume and pressure.

    - where the aortic valve opens and where it closes.

    - From the workspace, what is the end diastolic volume and what is the end systolic volume?

    - What is the stroke volume?

    - What is the cardiac output? How did you arrive at this answer (show formula)

    - What is the work done by the ventricle in one cycle (one heartbeat) if 1mmHg pressure = 133.32 Pa so that 1-liter x 1mmHg = 0.13332 joules (1ml x 1mmHg = 0.00013332 joules)?

- Recall that the left ventricle has an inflow, the mitral valve. and an outflow, the aortic valve. Cardiac valve malfunction occurs in one of two ways or in some combination of the two. These are stenosis and insufficiency. Insufficiency is also called incompetence or regurgitation. In valvular stenosis, the valve is narrowed and presents a higher-than-normal resistance to forward flow. In valvular incompetence, the valve is leaky and allows backflow. In mitral insufficiency, the backflow will occur during systole. As the ventricle contracts, it pushes blood back into the left atrium as well as forward into the aorta. In aortic insufficiency, the backflow occurs during diastole when the ventricle is relaxed (i.e. when ventricular pressure is less than arterial pressure). Some of the blood that was previously ejected into the aorta will flow back into the left ventricle. In our model of the left heart and systemic arteries, stenosis is modeled by increasing RMi (resistance of the mitral valve) and RAo (resistance of the aortic valve). Increasing either of these resistances simulates stenosis of the corresponding valve.

- For uncompensated aortic stenosis you will increase the resistance of the aortic valve from 0.01 mmHg/L/min to 1 mmHg/L/min.You have introduced a significant increase in resistance through the aortic valve (1000-fold). This increase was created suddenly. In reality, that is not what happens. Aortic stenosis is a progressive condition that starts by a small increase in resistance and develops into a greater increase incrementally and progressively. The heart compensates for the increasing resistance progressively as well. Like all muscle the cardiac muscle, faced with an increased resistance will hypertrophy (bigger, denser muscle) to maintain blood pressure and cardiac output. This hypertrophy will reduce the ventricular compliance. Ventricular compensation for the increased resistance (hypertrophy with decreased compliance) will require that the atrial pressures increase to fill the ventricle during diastole. In summary:

  - The ventricular muscle works against a larger afterload, which causes increased ventricular pressure. As in any muscle, the work against an extra load leads to hypertrophy of the muscle. This thickening of the muscle causes reduced ventricular compliance, i.e., thicker, stiffer muscle. Ventricular compliances, systolic and diastolic, will be reduced.

  - The reduction in diastolic compliance creates a problem for atrial emptying during diastole. Higher atrial pressures are needed to overcome the increased stiffness of the ventricles.

  - You have two forms of compensation to calculate: the reduced compliances of the ventricle and the increased atrial pressure necessary to fill a stiffened ventricle.
Calculating the necessary reduction in compliances is a bit tricky since pressure depends on compliance and compliance depends on pressure. This requires a reiterative calculation.

  - Start with all parameters reset to the original RAo = 0.01. For the normal aortic valve, assume that both the systolic and diastolic left ventricular compliances are inversely proportional to the maximum (systolic) left ventricular pressure. The constants of proportionality are determined by the normal values. So, 
CLVS=KSMax. LV Pressure and CLVD=KDMax. LV Pressure
Using the normal values for CLVS and CLVD, you can calculate the constants of proportionality, Ks and Kd. 
Then change the RAo from 0.01 to 1 to simulate aortic stenosis. This will render a new maximal LV pressure. Recalculate CLVS and CLVD after the resistance of aortic valve is incorporated into the algorithm (same constants of proportionality but new Max. LV pressure). Using the new CLVD and CLVS, run the program again and obtain new values for max. LV pressures. Then recalculate CLVS and CLVD. Reiterate the process until the results are constant. Clearly, a loop will automate this process. Also, you can reduce the number of iterations by averaging the results from one iteration to the next!

    - Hint: After about 5-20 iterations the compliances converge to stable values. I would recommend writing a script (for/loop) to find these values or spend the time running the model manually 5-20 times and do the calculations by hand, substituting the new values, etc. Writing the script to do this may take a little time but will be very instructive. Writing scripts to do your work is what you will need to do in the future as an engineer.

    - Report the new CLVD and CLVS in your submission

    - Now that the resulting stiffening of the cardiac muscle is accounted for by decreasing the compliances of the ventricle (first of two compensations),

      - calculate and report the stroke volume, cardiac output, work per beat and work per minute of the left ventricle with the aortic stenosis and the new compliances.

    - How did the aortic stenosis and subsequent compensation in muscular thickness affect these values?

    - After you have resolved the new values of CLVD and CLVS, use a trial and error or a “for/loop” to find the left atrial pressure that makes the cardiac output and stroke volume ± the cardiac output that existed before the stenosis (i.e. with normal aortic valve resistance of RAo = 0.01 mmHg/L/min).

      - What is required left atrial pressure to restore cardiac output? Hint: increase the left atrial pressures by increments of 0.2 mmHg until you have reached the desired stroke volume.

  - Once you have completed both compensation processes:

    - What are the left ventricular pressures (systolic and diastolic) and the arterial blood pressures? Why are the systolic pressures so different?

  - Compare the cardiac cycle plots when the aortic valve is normal and when it is stenosed and compensated (show your plots and describe the differences). What has happened to end systolic dPdV the elastance?
What does this change indPdV signify? In fact, what does the elastance measure?

  - How did the stenosis affect the work requirements of the heart?

    - How much, in percent, is the work of each heart stroke increased?

  - The normal cross-sectional area of the open aortic valve is variable from person to person (about 2.5 for very slight individuals-4 cm2 for large individuals). Assuming our model has an aortic valve area of 4 cm2 normally, using the Hakki formula, calculate the cross-sectional area of the aortic valve when the resistance through the valve is increased from 0.01 to 1.0 mmHg/(L/min). Mild stenosis exists when the area is > 1.5 cm2. Moderate stenosis is defined as an area of 1 to 1.5 cm2. Severe stenosis occurs when the area is diminished to <1 cm2. 
Chest pain, fainting or shortness of breath in people with aortic stenosis are indications for prompt valvular surgery as these signs signal an increased risk of sudden death.

- Next, we investigate mitral stenosis:

  - Let RMi = 1.0 mmHg/liter/min (Mitral Valve Stenosis). Observe (and report) the effects (before any compensation):

    - On stroke volume and cardiac output,

    - left ventricular blood pressures,

    - systemic arterial blood pressure,

    - Given what you see in these parameters, is compensation for these deficits mandatory?

    - Show plot of cardiac cycle

  - Note, however, that these are the uncompensated effects of this condition. We will address compensation:

    - To compensate for the deleterious effects of mitral stenosis, the left  atrial pressure must increase markedly (the ventricle is unimpeded and works well but is not being filled adequately)
With the Rmi = 1.0 mmHg/liter/min, increase the left atrial pressure in increments of 2-2.5 mmHg until the cardiac output is restored to ± normal.

    - What is this final atrial pressure? You have calculated the average systolic atrial pressure needed to fill the heart during atrial systole. Because the mitral stenosis represents a significant obstruction to flow, the left atrium hypertrophies (to increase “pushing” power) and dilates to accommodate more blood that is “dammed.” This affects the conductivity of the atria and atrial fibrillation usually ensues

      - look up atrial fibrillation and write a short description. Does the atrium contract once atrial fibrillation is established? If not, how can the atrium compensate for the lack of contraction?

    - The increased atrial pressure will be reflected into the pulmonary circulation and the pulmonary capillaries must have a higher pressure than the left atrium to maintain blood flow. Pressures >20 mmHg in these capillaries leads to transudation of fluid into the alveoli and inability to oxygenate. This is called cardiogenic pulmonary edema.

    - Based on the pressure gradient between left atrium and left ventricle, what statement can you make about the extent of mitral stenosis in this model (Rmi = 1.0 mmHg/liter/min) – see chat below

- Aortic insufficiency can be modeled here by increasing the parameter, AoBkflo (normal setting is zero), in the MATLAB code. Double clicking on the Simulink subroutine, QAo, shows why this works. This parameter simulates backflow, 
              Diastolic volume of aortic  flow=AoBkflo*(Paorta distolic-Pventricle diastolic)RAo 
Normally, flow through the aorta occurs only in systole. The “switch bloc” allows flow during diastole or in the equations we use:
QAo=SAoPLVt-PSatRAo                 SAo = 1 if PLV > Psa  (Ejection Phase of Systole)

SAo = 0  if PLV < PSa

In aortic insufficiency, the aortic valve cannot close completely in diastole.

If SAo ≠0 during diastole and since LV pressure is less than arterial diastolic pressure, flow re-entering the ventricle occurs.
This modification simulates backflow into the ventricle from the aorta during diastole. The model is very sensitive to aortic insufficiency as are we. If blood returns to the ventricle from the aorta during ventricular diastole, it does not go downstream to perfuse the body and organs. The left ventricle fills partially with blood it already has ejected in addition to blood from the left atrium. Cardiac output consists of blood that perfuses peripheral organs plus the blood that flows back into the ventricle during diastole. Intraventricular diastolic pressure is increases. This means the left atrium cannot empty adequately without a significant increase in left atrial pressure. To accommodate the extra volume (regurgitated blood from the aorta and the necessary cardiac output), the left ventricle compensates initially by increasing volume. To do this, there must be an increase in both the left ventricular diastolic compliance as well as the left atrial pressure. In addition, the force of contraction must be increased. This is accomplished by increasing the difference between the systolic ventricular compliance and the diastolic ventricular compliance. Increasing the difference in the diastolic and systolic ventricular compliances simulates increased capacitance of the ventricle & increased contractility.

  - Start by increasing AoBkflo from zero to 0.002. This small increase will have a tremendous effect on the flow dynamics. Remember that valvular regurgitation occurs because the valve never closes COMPLETELY, i.e. it is open “some.” If it were completely open, the result could be rapidly fatal and when that does happen (valvular necrosis and rupture), it constitutes a rare surgical emergency. By assigning a small value, we take the resistance of the aortic valve to backflow (normally infinite, hence no backflow) into account. Thus, assigning a value of 0.002 to AoBkflo, causes a small percentage of the diastolic pressure difference between aorta and ventricle to activate regurgitation back into the ventricle. Even with a small amount of regurgitation, the dynamic effects are impressive. Don’t forget to readjust your diastolic (starting) blood pressure.

  - Now repeat the process you used above in the case of aortic stenosis to determine the new end systolic and end diastolic ventricular compliances. Use the normal values of Ks & Kd previously found

    - Report the new CLVD and CLVS.

    - Are they increased or decreased?

    - Explain why they have changed in the direction the did.

    - Has the distance between them changed?

  - Now to complete the compensation of the aortic insufficiency, increase the atrial pressure by increments of 0.5 mmHg until the systolic blood pressure is in the normal range (about 102 to 107 mmHg). You will need a for/loop to do this effectively (or a lot of patience!)

  - Now that you have incorporated the compensations (changed compliances and increased atrial filling pressure), run the model again.

    - Don’t forget to readjust your diastolic (starting) blood pressure.

    - Note that the period of isovolumic contraction is now very short. Explain why that occurred.

    - As aortic insufficiency causes blood to backflow during diastole, there is a shortened period of isovolumic relaxation.

    - What is the stroke volume and how does it compare to the normal?

    - What is the work done per stroke?

    - How does this work/heartbeat compare to the normal? Why does the difference exist?

    - What happens to the aortic blood pressure? Why?

    - What happens to the pulse pressure? Why?

    - Without any compensation what is the difference in stroke volume between the simulations with and without aortic insufficiency (ml/beat)?

    - Look at the diastolic arterial blood pressure. Explain the change in blood pressure in terms of energies.

    - Look up “water hammer pulse” and Quincke’s pulse sign. Explain these clinical signs by what you see in the model.

    - Graph the aortic and ventricular blood flows. Why do you see negative flows seen in the graph of the aortic blood flow.

- Finally, rheumatic fever, an autoimmune reaction to a β- streptococcus infection (strep throat) can result in multi-valvular disease. The combination is often aortic stenosis with mitral stenosis. The combination of the mechanisms necessary to compensate for these valvular abnormalities: decreased ventricular compliance and very large increases in atrial pressure engender severe congestive heart failure. Without modeling this problem, I think you can appreciate the severity of the problem.